SOP for Looming system developed by Zengyou

03/17/2026

Hardware and Software:

- Computer with 2 external monitors

- USB Camera x1 with holding arm

- Laser source: Doric LD Fiber Light Source

- Customized IO box (Arduino Uno)

- Looming chamber: 40x40x32.5cm

- Customized Bonsai-RX workflow

2

1

3

4

5

6

Experimental Protocol:

- Habituate mice to the testing chamber for 15 min per day in 3 consecutive days.

Start from step 2.2 to 3 without clicking “Start” in step 2.5 on Bonsai UI panel.

2. Behavioral Testing (2 Days, Counterbalanced Design)

- Day 1: No Optogenetic stimulation (control)

- Day 2: With Optogenetic stimulation

Note: Optogenetic stimulation begins at looming onset and is maintained throughout the remainder of the recording.

2.1 Light Source Setup (if Optogenetic stimulation needed)

- Turn on the Doric light source (638 nm for Halo or ChrimsonR).

- Press the control knob to select “External TTL” mode.

- Adjust the knob to set the output to 79 mA (~10 mW laser power).

Note: 1. Verify laser power with meter as needed; 2. Use Doric Studio for complex simulations.

2.2 Turn on the computer and all monitors.

2.3 Open the Bonsai workflow: ZY_Loomstim_V6_manual_2020316.bonsai

2.4 Pre-recording setup:

- Run the Bonsai workflow.

- If the UI does not appear automatically, open it manually.

- Set the following parameters: Data folder, User, Experiment ID, and Mouse ID

2.5 Behavioral Recording Procedure

- Place the mouse into the chamber (with patch cable connected if needed) and click “Start”.

- After a 10-minute baseline, the “start_stim” button will appear for looming stimulation.

- Deliver stimulation only when the mouse is in the corner farthest from the safety shelter.

- Recording will automatically stop 30 seconds after stimulation

Stimulation parameters:

Initial size at 2° for 250 ms, followed by expansion to 50° over 250 ms, then maintained at 50° for 500 ms. This sequence is repeated 10 times with an inter-stimulus interval (ISI) of 1.6 s.

3. Post-Trial Handling

- Return the mouse to its home cage.

- Clean the chamber with 70% ethanol.

- Allow the chamber to dry before testing the next mouse.